# DiaWeb

Ensuring Timely Medication, Accurate Monitoring, and Better Diabetes Care.

Documentation

February 2026

# Table of Contents

1. System Overview .......................................... 3

2. Features & Capabilities .................................. 4

3. System Architecture ...................................... 6

4. Hardware Components ...................................... 8

5. Software Components ...................................... 10

6. Installation Guide ....................................... 12

7. User Manual .............................................. 18

8. API Documentation ........................................ 24

9. Troubleshooting .......................................... 28

10. Maintenance & Updates ................................... 32

Appendix A: Configuration Reference ......................... 34

Appendix B: Firebase Rules .................................. 36

Appendix C: Glossary ........................................ 38

# 1. System Overview

## 1.1 Introduction

DiaWeb is a comprehensive diabetes management system that combines a modern web application with IoT hardware to provide patients with real-time health monitoring, medication tracking, and physical notification alerts.

## 1.2 Purpose

The system is designed to:

• Help diabetic patients track their blood glucose levels consistently

• Ensure medication adherence through timely reminders

• Provide physical alerts via IoT buzzer for critical notifications

• Maintain comprehensive health records accessible anytime, anywhere

• Generate visual reports and calendars for healthcare provider consultations

## 1.3 Target Users

• Type 1 and Type 2 diabetic patients

• Healthcare providers monitoring patient data

• Caregivers managing patient medication schedules

• Family members tracking patient health

# 2. Features & Capabilities

## 2.1 Web Application Features

| Feature | Description |
| --- | --- |
| Blood Glucose Monitoring | Track readings with type (fasting, before/after meal, bedtime), automatic alerts for high (>180) and low (<70) values |
| Medication Management | Schedule medications with dosage, time, and frequency; mark as taken; view adherence |
| Custom Reminders | Set time-based reminders for any health-related task |
| Medication Calendar | Interactive calendar showing medication history with daily details |
| Statistics Dashboard | View average glucose, last reading, and medication adherence |
| User Authentication | Secure login with Firebase Authentication |
| Responsive Design | Works on desktop, tablet, and mobile devices |
| Real-time Sync | Data synchronized across all devices via Firebase |

## 2.2 Hardware Features

| Feature | Description |
| --- | --- |
| Physical Notifications | Piezo buzzer plays distinct patterns for different alert types |
| WiFi Connectivity | 2.4GHz WiFi for Firebase real-time monitoring |
| 24/7 Operation | Continuous monitoring with auto-reconnect capability |
| Low Power | Efficient power consumption for always-on operation |
| Visual Feedback | Built-in LED indicates system status |
| Multiple Patterns | 4 different buzzer patterns for medication, reminders, high/low glucose |

## 2.3 Alert System

The system provides multi-layered notifications:

| Alert Type | Web Notification | Buzzer Pattern |
| --- | --- | --- |
| Medication Time | On-screen popup | 3 short beeps |
| Custom Reminder | On-screen popup | 2 long beeps |
| High Glucose (>180) | Warning alert | 4 medium beeps |
| Low Glucose (<70) | Danger alert | 6 fast urgent beeps |

# 3. System Architecture

## 3.1 High-Level Architecture

DiaWeb follows a three-tier architecture consisting of the presentation layer (web application), data layer (Firebase), and notification layer (ESP8266 hardware).

## 3.2 System Components

┌─────────────────────────────────────┐

│      WEB APPLICATION                │

│  (HTML + CSS + JavaScript)          │

└──────────────┬──────────────────────┘

│

▼

┌─────────────────────────────────────┐

│     FIREBASE CLOUD                  │

│  • Authentication                   │

│  • Realtime Database                │

│  • Hosting (optional)               │

└──────────────┬──────────────────────┘

│

▼

┌─────────────────────────────────────┐

│     ESP8266 HARDWARE                │

│  • WiFi Connection                  │

│  • Firebase Listener                │

│  • Piezo Buzzer                     │

└─────────────────────────────────────┘

## 3.3 Data Flow

1. User logs into web application

2. User adds blood glucose reading or medication

3. Data saved to Firebase Realtime Database

4. System evaluates if notification is needed

5. If needed, notification sent to /notifications/{userId}/latest

6. ESP8266 detects new notification (checks every 5 seconds)

7. ESP8266 plays appropriate buzzer pattern

8. ESP8266 acknowledges notification in Firebase

# 4. Hardware Components

## 4.1 ESP8266 Microcontroller

| Specification | Details |
| --- | --- |
| Model | ESP8266 (NodeMCU, Wemos D1 Mini, or similar) |
| WiFi | 802.11 b/g/n, 2.4GHz only |
| Operating Voltage | 3.3V |
| Input Voltage | 5V (via USB) |
| GPIO Pins | 11 (D0-D10) |
| Flash Memory | 4MB |
| Clock Speed | 80MHz (160MHz capable) |
| Power Consumption | ~70mA (active), ~20µA (deep sleep) |

## 4.2 Piezo Buzzer

| Specification | Details |
| --- | --- |
| Type | Passive Piezo Buzzer (recommended) or Active Buzzer |
| Operating Voltage | 3-5V |
| Frequency Range | 1000-3000 Hz |
| Sound Level | 85-95 dB @ 10cm |
| Connection | Positive to D1 (GPIO5), Negative to GND |

## 4.3 Wiring Diagram

Basic Connection:

ESP8266          Piezo Buzzer

├─ D1 (GPIO5) ────── Positive (+)

└─ GND ───────────── Negative (-)

Optional: Add 100Ω resistor in series for volume control

# 5. Software Components

## 5.1 File Structure

| File | Purpose |
| --- | --- |
| index.html | Main web application structure and layout |
| styles.css | Modern medical UI styling with responsive design |
| script.js | Firebase integration, data management, notifications |
| ESP8266_Buzzer.ino | Arduino code for ESP8266 microcontroller |
| README.md | Project overview and quick start guide |
| ESP8266_SETUP_GUIDE.md | Detailed hardware setup instructions |
| WIRING_DIAGRAMS.md | Visual wiring reference with ASCII diagrams |
| CALENDAR_FEATURE.md | Calendar feature documentation |
| FIREBASE_LIBRARY_FIX.md | Library installation troubleshooting |

## 5.2 Technology Stack

### Web Application:

• HTML5 - Structure and semantic markup

• CSS3 - Modern styling with CSS Grid, Flexbox, animations

• JavaScript (ES6+) - Application logic and Firebase integration

• Firebase SDK 9.22.0 - Authentication and Realtime Database

### Hardware:

• Arduino IDE - Development environment

• C++ - Programming language for ESP8266

• Firebase ESP Client Library - Firebase integration

• ESP8266 Core 3.0+ - Board support package

## 5.3 Firebase Structure

Database Path Structure:

/users/

└─ {userId}/

├─ name

├─ email

└─ createdAt

/bloodSugar/

└─ {userId}/

└─ {readingId}/

├─ value

├─ type

├─ timestamp

├─ date

└─ time

/notifications/

└─ {userId}/

├─ latest/

│   ├─ id

│   ├─ type

│   ├─ message

│   └─ timestamp

├─ history/

└─ acknowledged/

# 6. Installation Guide

## 6.1 Prerequisites

### Hardware:

• ESP8266 development board (NodeMCU recommended)

• Passive piezo buzzer (5V tolerant)

• USB cable (Micro-USB)

• Jumper wires

• Computer with internet connection

### Software:

• Modern web browser (Chrome, Firefox, Edge)

• Arduino IDE 1.8.13 or higher

• Firebase account (free tier)

• Text editor (VS Code, Sublime, Notepad++)

## 6.2 Firebase Setup

### Step 1: Create Firebase Project

1. Go to console.firebase.google.com

2. Click "Add project"

3. Enter project name: "DiaWeb"

4. Disable Google Analytics (optional)

5. Click "Create project"

### Step 2: Enable Authentication

1. In Firebase Console, click "Authentication"

2. Click "Get started"

3. Go to "Sign-in method" tab

4. Enable "Email/Password"

5. Click "Save"

### Step 3: Create Realtime Database

1. Go to "Realtime Database" in left menu

2. Click "Create Database"

3. Choose your region

4. Start in "Test mode"

5. Click "Enable"

### Step 4: Configure Database Rules

See Appendix B for complete Firebase Rules configuration

## 6.3 Web Application Setup

### Step 1: Get Firebase Credentials

1. Go to Project Settings (gear icon)

2. Scroll to "Your apps" section

3. Click web icon (</>)

4. Register app with nickname: "DiaWeb"

5. Copy the firebaseConfig object

### Step 2: Configure Web App

1. Open script.js in text editor

2. Find the firebaseConfig object (around line 10)

3. Replace with your Firebase credentials

4. Save the file

### Step 3: Test Web Application

1. Open index.html in web browser

2. Click "Create Account"

3. Register with email and password

4. Login and test features

## 6.4 ESP8266 Setup

### Step 1: Install Arduino IDE

1. Download from arduino.cc

2. Install on your computer

3. Open Arduino IDE

### Step 2: Install ESP8266 Board Support

1. File → Preferences

2. Add Board Manager URL:

http://arduino.esp8266.com/stable/package_esp8266com_index.json

3. Tools → Board → Boards Manager

4. Search "esp8266"

5. Install "esp8266 by ESP8266 Community"

### Step 3: Install Firebase Library

1. Sketch → Include Library → Manage Libraries

2. Search "Firebase ESP Client"

3. Install "Firebase Arduino Client Library for ESP8266 and ESP32" by Mobizt

### Step 4: Hardware Connection

1. Connect Buzzer + to D1 (GPIO5)

2. Connect Buzzer - to GND

3. Connect ESP8266 to computer via USB

### Step 5: Configure Code

1. Open ESP8266_Buzzer_Fixed.ino

2. Update WiFi credentials (SSID and password)

3. Update Firebase credentials (API Key, Database URL)

4. Update User credentials (email and password)

5. Verify pin configuration (BUZZER_PIN = D1)

### Step 6: Upload Code

1. Select Board: Tools → Board → NodeMCU 1.0

2. Select Port: Tools → Port → (your COM port)

3. Click Upload button

4. Wait for "Done uploading"

### Step 7: Monitor Serial Output

1. Open Serial Monitor (Tools → Serial Monitor)

2. Set baud rate to 115200

3. Verify connection messages

4. Listen for welcome beep