# DiaWeb - Complete Troubleshooting Guide

ESP8266 Notification System & Web Application

# Table of Contents

1. Common ESP8266 Errors

2. Firebase Connection Issues

3. Library Installation Problems

4. WiFi Connection Errors

5. Buzzer Not Working

6. Web Application Issues

7. Firebase Database Errors

8. Calendar Feature Problems

9. Quick Reference Guide

10. Contact & Support

# 1. Common ESP8266 Errors

## Error: "FirebaseESP8266.h: No such file or directory"

Problem: Old library name used in code.

Solution:

1. Open Arduino IDE Library Manager

2. Search for "Firebase ESP Client"

3. Install "Firebase Arduino Client Library for ESP8266 and ESP32" by Mobizt

4. Use the updated code file: ESP8266_Buzzer_Fixed.ino

## Error: "Board not found on selected port"

Problem: Computer cannot detect ESP8266 board.

Solutions:

• Install CH340 or CP2102 USB drivers (search for your board model)

• Try a different USB cable (some cables are charge-only)

• Try a different USB port on your computer

• Press and hold FLASH button during upload (on some boards)

• Check Device Manager (Windows) or ls /dev/tty* (Mac/Linux) to verify port

## Error: "Compilation error"

Problem: Code syntax errors or missing libraries.

Solutions:

• Ensure ESP8266 board support is installed (version 3.0.0 or higher)

• Verify all libraries are installed correctly

• Check for typos in configuration values (WiFi SSID, API keys, etc.)

• Make sure you're using the correct Arduino IDE version (1.8.13 or higher)

# 2. Firebase Connection Issues

## Error: "Firebase connection failed"

Problem: ESP8266 cannot connect to Firebase.

Checklist:

| Configuration | Correct Format | Common Mistakes |
| --- | --- | --- |
| API_KEY | AIzaSy... | Using Database Secret instead |
| DATABASE_URL | https://project.firebaseio.com | Missing https:// or wrong URL |
| USER_EMAIL | your@email.com | Wrong email or typo |
| USER_PASSWORD | yourpassword | Wrong password |

How to Get Credentials:

1. Go to Firebase Console (console.firebase.google.com)

2. Select your DiaWeb project

3. Click Gear icon → Project Settings

4. Under "General" tab, copy "Web API Key"

5. Go to Realtime Database to get Database URL

6. Use the same email/password you use to login to DiaWeb

## Error: "Authentication failed"

Problem: Login credentials are incorrect.

Solutions:

• Verify email is correct (case-sensitive)

• Verify password is correct

• Try logging into DiaWeb web app to confirm credentials work

• Create a new account if needed

• Check Firebase Console → Authentication to see if Email/Password is enabled

# 3. Library Installation Problems

## Problem: "Library not found in Library Manager"

Solution - Manual Installation:

1. Go to: https://github.com/mobizt/Firebase-ESP-Client

2. Click green "Code" button → Download ZIP

3. In Arduino IDE: Sketch → Include Library → Add .ZIP Library

4. Navigate to Downloads and select the ZIP file

5. Restart Arduino IDE

## Problem: "Library conflicts or version errors"

Solution:

1. Remove old Firebase libraries:

- Go to: Documents/Arduino/libraries/

- Delete folders: FirebaseESP8266, FirebaseArduino, etc.

2. Install fresh copy of Firebase ESP Client

3. Restart Arduino IDE

# 4. WiFi Connection Errors

## Error: "WiFi connection failed"

Common Causes:

| Problem | Solution |
| --- | --- |
| Wrong SSID or Password | Double-check credentials (case-sensitive) |
| 5GHz WiFi Network | ESP8266 only supports 2.4GHz - switch to 2.4GHz network |
| Hidden Network | Make SSID visible or add manually |
| MAC Filtering Enabled | Add ESP8266 MAC address to router |
| Too Far from Router | Move ESP8266 closer to WiFi router |
| Special Characters in Password | Use simple alphanumeric password |

## How to Check WiFi Signal

Add this code to Serial Monitor output:

Serial.print("WiFi Signal Strength: ");

Serial.println(WiFi.RSSI());

Good signal: -50 to -60 dBm

Fair signal: -60 to -70 dBm

Weak signal: -70 to -80 dBm (may have issues)

# 5. Buzzer Not Working

## Problem: No sound from buzzer

Troubleshooting Steps:

### Step 1: Check Wiring

• Buzzer Positive (+) connected to D1 (GPIO5)

• Buzzer Negative (-) connected to GND

• Connections are secure and not loose

### Step 2: Test Buzzer Polarity

• Try reversing the buzzer connections (swap + and -)

• Some buzzers are not polarity-sensitive

### Step 3: Test Buzzer Separately

• Connect buzzer to 3.3V and GND directly

• If no sound, buzzer may be broken - try different buzzer

### Step 4: Check Code Pin Configuration

• Verify BUZZER_PIN is set to D1

• Try different pin (D2, D5, D6) and update code

## Problem: Buzzer too quiet

Solutions:

• Remove any resistors in series with buzzer

• Use transistor amplifier circuit (see wiring diagrams)

• Try a different (louder) buzzer

• Increase frequency values in buzzer patterns

# 6. Web Application Issues

## Problem: Cannot login to DiaWeb

Solutions:

1. Clear browser cache and cookies

2. Try different browser (Chrome, Firefox, Edge)

3. Check Firebase credentials in script.js are correct

4. Verify Firebase Authentication is enabled in Firebase Console

5. Try creating a new account

## Problem: Data not saving to Firebase

Checklist:

• Check browser console for errors (Press F12)

• Verify you are logged in

• Check Firebase Database Rules allow write access

• Check internet connection

• Verify Firebase configuration in script.js

## Problem: ESP8266 not receiving notifications

Debugging Steps:

1. Check Serial Monitor on ESP8266 for connection status

2. Verify User ID matches in both web app and ESP8266 code

3. Open Firebase Console → Realtime Database

4. Navigate to /notifications/YOUR_USER_ID/latest

5. Manually add test notification to see if ESP8266 detects it

6. Check Firebase Database Rules allow read access to notifications

# 7. Firebase Database Errors

## Error: "Permission Denied"

Problem: Firebase Database Rules are too restrictive.

Solution - Update Database Rules:

1. Go to Firebase Console → Realtime Database

2. Click "Rules" tab

3. Replace with the correct rules (see documentation)

4. Make sure notifications path has read: true for ESP8266 access

5. Click "Publish"

## Error: "Database not found"

Problem: Realtime Database not created.

Solution:

1. Go to Firebase Console

2. Select your project

3. In left menu, click "Realtime Database" (NOT Firestore)

4. Click "Create Database"

5. Choose location closest to you

6. Start in Test mode (you can change rules later)

# 8. Calendar Feature Problems

## Problem: Calendar not showing medication data

Solutions:

• Refresh the page and reopen calendar

• Check Firebase database has medicationIntake data

• Verify date format matches between calendar and database

• Check browser console for JavaScript errors

## Problem: Calendar modal won't open

Solutions:

• Check browser console for errors

• Ensure script.js is loaded correctly

• Try hard refresh: Ctrl+Shift+R (Windows) or Cmd+Shift+R (Mac)

• Clear browser cache

# 9. Quick Reference Guide

## ESP8266 Serial Monitor Messages

| Message | Meaning |
| --- | --- |
| WiFi Connected! | ✓ WiFi working correctly |
| Firebase connected! | ✓ Firebase authentication successful |
| Monitoring for notifications... | ✓ System ready, waiting for alerts |
| NEW NOTIFICATION RECEIVED! | ✓ Notification detected, buzzer will play |
| WiFi connection failed! | ✗ Cannot connect to WiFi - check SSID/password |
| Firebase connection failed! | ✗ Cannot connect to Firebase - check credentials |
| Failed to get notifications | ✗ Cannot read Firebase - check rules/permissions |

## Buzzer Pattern Reference

| Pattern | Notification Type |
| --- | --- |
| 3 short beeps | Medication reminder |
| 2 long beeps | Custom reminder |
| 4 medium beeps | High blood sugar (>180 mg/dL) |
| 6 fast urgent beeps | Low blood sugar (<70 mg/dL) - URGENT! |
| Rising tones (Do-Re-Mi) | Welcome beep on startup |

## Essential File Checklist

• index.html - Main web application

• styles.css - Styling for web app

• script.js - JavaScript with Firebase integration

• ESP8266_Buzzer_Fixed.ino - Arduino code for ESP8266

• All files must be in same folder (except Arduino code)

# 10. Contact & Support

## Before Asking for Help

1. Check Serial Monitor output for error messages

2. Verify all configuration values are correct

3. Test each component separately (WiFi, Firebase, Buzzer)

4. Review relevant section in this guide

5. Take screenshots of error messages

## Useful Resources

• Firebase Console: https://console.firebase.google.com

• Arduino IDE Downloads: https://www.arduino.cc/en/software

• ESP8266 Community Forums: https://www.esp8266.com

• Firebase ESP Client Library: https://github.com/mobizt/Firebase-ESP-Client

## Diagnostic Checklist

When reporting issues, include:

□ Arduino IDE version

□ ESP8266 board type (NodeMCU, Wemos, etc.)

□ Library versions installed

□ Serial Monitor output (complete log)

□ Firebase project region

□ Browser console errors (for web app issues)

□ Steps already tried to fix the issue

# Appendix: Configuration Template

### ESP8266 Configuration (ESP8266_Buzzer_Fixed.ino):

// WiFi Settings

#define WIFI_SSID "YOUR_WIFI_NAME"

#define WIFI_PASSWORD "YOUR_WIFI_PASSWORD"

// Firebase Settings

#define API_KEY "AIzaSy..."

#define DATABASE_URL "https://your-project.firebaseio.com"

#define USER_EMAIL "your@email.com"

#define USER_PASSWORD "yourpassword"

### Web App Configuration (script.js):

const firebaseConfig = {

apiKey: "YOUR_API_KEY",

authDomain: "YOUR_AUTH_DOMAIN",

databaseURL: "YOUR_DATABASE_URL",

projectId: "YOUR_PROJECT_ID",

storageBucket: "YOUR_STORAGE_BUCKET",

messagingSenderId: "YOUR_MESSAGING_SENDER_ID",

appId: "YOUR_APP_ID"

};

Last Updated: February 10, 2026